4. Always use  the battery  charger supplied  by the  manufacturer  to avoid any

damage  to  the   rechargeable   battery.  This   device  can  be  used   during

charging.

NOTICE  OF USE

2G / 3G / 4G / 5G (3300 ~ 3700MHz)

Cell Phone Detector

1. Never  remove or  change the  antenna.

2. Unauthorized  repair  or dismantle  of this  device will  void all  the warranties.

3. Avoid  water.

4. Do not  store this  device in  an excessively  hot place.

5. Avoid  knocks or  dropping this  device.

User's Manual

Thank  you  for  purchasing    2G/3G/4G Cell  Phone  Detector.  Please

first  read over  this manual  for  proper use,  save  this manual  and keep  it handy.

SPECIFICATION

Power

1. Built-in  rechargeable   battery

2. Switching   power adaptor

GENERAL

Battery  life

About  4 hours  after  full charge

This device  is made  for detecting  2G

/ 3G / 4G / 5G (3300~3700MHz) cell  phones radio  waves  in a place.  This device  also can detect WiFi, Bluetooth, wireless  camera from  50 MHz - 6.0 GHz.

Detecting  frequency   band

1.  2G, 4G, 5G(3300~3700MHz) WiFi, Bluetooth

2.  CDMA

3.  3G:  WCDMA  2100  (UMTS)

Detecting  distance

1.  3G  (WCDMA)  : up  to 15 meters

2.  GSM  : up  to 4 meters

3.  CDMA , 4G, 5G(3300~3700MHz)  :  up to 4  meters

This device  is special  designed

insensitive  with 2-way  raido.  So, it

will not  be interfered  with 2-way

Dimension

Weight

L 150  x W 80  x T 30  mm

About  310 g

Network indicator

radio easily.  This  feature makes

Power

this device  well suited  to use  in

the places  not allowed  for

cellphone  communication,  yet  the

security  person communicate  via

2-way radio.

Sensitive level

(Earphone)

1. Visible  and  audible  alarm

Warning  mode

2. Visible  alarm,  silent  via  earphone

Operation   temperature

Humidity

0 - 40  centigrade

5% ~  80%

It has  network indication  (LED)  of

2G/Camera  & 3G individually  to

show the  type of detected  radio

waves. * 5G / 4G / WiFi / Bluetooth are indicated at 2G / 4G / Cam.

* Specifications may change without notice

WARNING

Acrylic seat

Use this device as an auxiliary, supplemental help or aid to prevent the risks caused by hidden camera, cellular phone or other wireless devices. This

device does not take the place of all the supervisions. Performance of this Radio frequency (RF) product will be affected by the circumstance of use.

The producer and marketing group accepts no liability for any loss or damage by malfunction or misuse.

When detected  radio  wave, the  network indicator  (LED)  will blink  in red. This

device  has sensitivity  tuner  for 2G/4G/Camera  & 3G individually  for  adjusting

the sensitivity  and the  detection  distance.

COPYRIGHT

No part  of this  manual may  be reproduced,  transmitted, transcribed,  or translated  into any  language in  any form  by any  means, electrical  or

mechanical, including  photocopying, recording,  or information and  retrieval systems,  without the express  written permission  of manufacturer.

Products mentioned in this  manual are for identification purposes  only. All brand names appearing in  this manual may or may  not be registered

trademarks or copyrights of their respective companies.  The manufacturer reserves the right to change or modify the specifications  of any one or

all of these products as deemed necessary at any time without prior notification. The manufacturer may further revise this manual from time to time

without prior notice.

This device  has built-in  rechargeable  battery  that can  last for  4 hours

continuous  use. When  battery  power runs down,  the battery  low warning

indication  will light  up in red  to warn the  user.

Copyright 2000-2023. All rights reserved.

PK1A

01.2023

Made in Taiwan.

HOW TO  OPERATE

DETECTING  INDICATION

1. Press   down  the   On-Off   switch

blinking

in red

and  the   power  indicator   in  the

front side  will light  up in green.

2G/Camera/4G/5G/WiFi/Bluetooth detecting

indication:

On-Off switch    Sensitivity turner

This  picture shows this device detects  2G / Camera / 4G

5G / WiFi / Bluetooth radio   wave,  the network indicator

of 2G / 4G / Camera  is blinking  in red.

Battery charger  jack

2. Then  unbend   the  antennas,   please   always   keep  the  antennas   in  vertical

And the  3 LEDs of  2G/4G/Cam  sensitive level  indicator

light up.

straight  for better  result.

3G detecting  indication:

blinking

in red

3. Please be  noted  that  the  sensitivity  adjustment  of  2G/Camera/4G/5G/WiFi

/Bluetooth & 3G  are different.

Setting  the sensitivity  level  of 2G / Camera / 4G / 5G (3300~3700MHz) /  WiFi / Bluetooth:

This picture  shows  this device  detects 3G  radio wave,

the network  indicator  of 3G is  blinking in  red.

And the  3 LEDs of  3G sensitive   level indicator  light  up.

When detect  the cell  phone radio  wave, you  can reduce  the sensitivity  level

gradually  to approach  the signal  source.

1. Turn  the sensitivity  tuner  to +

(plus)  side until  the first yellow

LED light  up.

Yellow LED

light up

BATTERY  LOW WARNING

There  is a battery  low  warning  indication  on the  front side.  When  battery  runs

down,  the  battery  low  warning   LED  will  light  up  in  red.  Please  connect  the

battery  charger to  charge the  battery.

2. Turn  the sensitivity  tuner  to  -

(minus)  side to let  the yellow

LED put  out.

Yellow LED

put out

SILENT  DETECTION

Note: Over  sensitive  or too low  sensitive  setting should  affect  the

detection  ability.

There  is an earphone  jack  in the right  side  of front view.

Connect  the earphone  to detect  by earphone  hearing.

Setting  the sensitivity  level  of 3G:

ABOUT BATTERY

Turn the  sensitivity  tuner to  + (plus) side  at the maximum  place.  If one or

two LEDs  (sensitive  level indicator)  light  up, this  is OK.

1.  When  the  battery  low warning   LED light  up  in  red.  Connect  the  switching

power  adaptor  to continue  the  detecting  and  to recharge  the  battery  at the

same time.

2.  It  will  take  about  7  hours   to  full  charge  the  battery   under  POWER  OFF

condition.   If  recharge  under  POWER   ON  condition,  it  will  take   about  16

hours to  full charge  the battery.

3.  If you are  not going  to use  this device  for a long  time, please  set the  On/Off

switch  at Off and  check the battery  power  once three  months.